- Downloading and Installing tk8500 
on Microsoft Windows Systems

Version 0.9

tk8500 has been tested on a 1400 MHz AMD Athlon® based computer running Microsoft Windows 98. and Tcl/Tk version 8.3, patchlevel 8.3.3.  This is experimental software and probably contains defects. It remains under development. When available, newer versions of tk8500 will be made available for download at this web site.

- Downloading and installing tk8500 for Windows

- Download and run the free tcl/tk software from:  http://prdownloads.sourceforge.net/tcl/tcl832.exe .

It will ask you to choose a directory in which to install the tcl/tk software. You must install it in a new folder named Tcl on the C partition, i.e.,

C:\Tcl

- Create another installation folder, named C:\tk8500. It will be used for the tk8500 program and data files.

- Download the zipped version of tk8500, tk85-09.zip into the C:\tk8500 folder you created earlier.

- Unzip the file into the C:\tk8500 folder after downloading it. (Note to Windows XP users: Be careful to extract the zip file into the correct folder. Windows XP defaults to the wrong folder and you must explicitly choose the correct folder.)

- Remove the tk8500.ini configuration file if you have one left over from a previous installation.

- You must reboot Windows before using tk8500 for the first time.

- To start tk8500, run the C:\tk8500\tk8500.bat batch file.

- Notes

- This installation procedure changed and is now the same for Windows XP, Windows 98, and Windows ME. The older instructions required editing the autoexec.bat file, a file not used in Windows XP.

- If you create a shortcut to C:\tk8500\tk8500.bat, the shortcut works best if it is set up to run minimized and close on exit.

- Memory channel data files created for tk8500 version 0.4 and earlier will not work with this version. To use these data files, you must add a new Volume field (located before the Label field) to each line. It is easy to insert a new data column using Gnumeric, Excel, Star Calc, or other spreadsheet program. Don't forget to to save the file in .csv format.

- Before using tk8500, be sure the IC-R8500's "initial set mode" options are set as follows:

  - CIV ADDR = 4AH

  - CIV BAUD = AUTO

  - CIV TRAN = OFF

  - CIV 731 = OFF

See pp. 31 - 32 in the IC-R8500 instruction manual to learn how to set them.

- Power your IC-R8500 receiver on before starting tk8500 or else the program will freeze.

- Overview

- tk8500 is open source software designed to control and program the ICOM IC-R8500 receiver. An introductory article about this software appeared in April 2002 Monitoring Times magazine. tk8500 was favorably reviewed by Peter Bond in Britain's Short Wave Magazine, November 2002.

- Though tk8500 is intended chiefly for Linux, MacOS X, and BSD users, it will work on Microsoft Windows (95 and later), too.

- tk8500 is distributed free of charge, but it is neither shareware nor in the public domain. tk8500 is a copyrighted work released under the terms of the GNU General Public License as published by the Free Software Foundation. tk8500 is distributed in the hope that it will be useful, but WITHOUT ANY WARRANTY; without even the implied warranty of MERCHANTABILITY or FITNESS FOR A PARTICULAR PURPOSE. See the GNU General Public License for more details.

- Most IC-R8500 controls are available in tk8500's Main Controls, Adjustments, and Secondary Controls windows. Less frequently used parameters may be accessed using the Scan Options pulldown menu, which may be "torn off" and used as a separate window (fig. 3).

- You can only scan one bank at a time while operating the IC-R8500 using the radio's front panel controls. tk8500 lets you scan multiple memory channel banks. You can designate a special volume setting to be used when receiving selected frequencies (no sound card required). The louder audio can alert you signals on important frequencies while scanning or can compensate for undermodulated transmitters.

- tk8500 can be used to load the IC-R8500 memories or read memory data from the radio and store it in a file. Channels may be sorted by frequency or by label, within any bank or all banks.

- Data files are in csv (comma-separated values) format, so users can create or edit them using spreadsheet or text editor programs.

- tk8500 software provides a facility to log and timestamp active frequencies and relative signal strengths to a log file in csv format. For example:

2002/01/19,18:06:28,159.15000,fm,99,"KANE"

2002/01/19,18:07:56,460.52500,fm,135,"KENDL_P1"

2002/01/19,18:07:59,154.86000,fm,0,"DE_KALB"

2002/01/19,18:08:03,154.86000,fm,89,"DE_KALB"

2002/01/19,18:08:05,154.86000,fm,89,"DE_KALB"

2002/01/19,18:08:25,155.47500,fm,84,"ISPERN"

2002/01/19,18:13:23,460.52500,fm,0,"KENDL_P1"

2002/01/19,18:14:02,159.15000,fm,0,"KANE"

- Memory Channel Files

- tk8500 can read files containing memory channel data. Using the mouse, you can click on a particular entry and tk8500 will tune the radio and set the mode, skip and select flags accordingly (fig. 4).

- You can also download the memory data from a file to the IC-R8500, thus programming its memories.

- A memory channel data file must be formated in csv (comma-separated values) format. The first line of the file should be this heading:

  - Bank,Ch,MHz,Step,Mode,Skip,Select,Atten,Volume,Label

- All the other lines should contain these fields:

Bank number (0 - 19)

Channel number (0 - 99)

Frequency in MHz

Step

Mode (am, amn, amw, usb, lsb, cw, cwn, fm, fmn, wfm)

skip (optional)

select (optional)

attenuation (optional: 0, 10, 20, 30)

Volume (optional: vol2 )

Label (an optional 8-char name which should not

contain a comma or quotation marks.)

Note (an optional variable length comment which should not

contain a comma or quotation marks.)

This is a sample memory data file:

Bank,Ch,MHz,Step,Mode,Skip,Select,Atten,Volume,Label,Note

0,1,460.525,12.5,FM,,select,,vol2,"KNCOM P1","Kendall County Sheriff 1"

0,2,460.575,9,FM,,select,,vol2,"KNCOM F1","Kendall County Fire 1"

0,3,460.375,12.5,FM,,select,20,vol2,"KNCOM P2","Kendall County Sheriff 2"

0,4,462.975,12.5,FM,,select,,vol2,"KNCOM F2","Kendall County Fire 2"

0,5,155.475,5,FM,,select,,vol2,ISPERN

0,6,154.71,10,FM,,select,,,ExecProt,"IL Excecutive Protection"

0,7,155.46,5,FM,,select,,,"ISP HF4"

0,8,145.17,5,FM,,,,,IHARC,"IH repeater Naperville"

0,9,155.58,5,FM,,select,10,vol2,Oswego,"Oswego Police local comms"

0,10,146.52,5,FM,,,,,SIMPLEX,"2 meter simplex"

0,11,148.6,5,FM,,select,,,"ARMY JL","Army Elwood firing range"

0,12,154.95,5,FM,,select,,vol2,CPAT,"CPAT police surveillance"

0,13,145.77,10,FM,skip,select,,,CARMA,"CARMA 2m simplex frequency"

0,14,419.65,12.5,FM,skip,,,,"USPS INS"

0,15,52.525,10,FM,skip,,,,"6M smplx"

You can use a separate text editor or spreadsheet program to create, edit, and print memory channel files. You cannot create or edit memory channel data files within tk8500.

- Downloading and Installing tk75 
on Microsoft Windows Systems

Version 0.4

tk75 has been tested on a 1400 MHz AMD Athlon® based computer running Microsoft Windows 98. and Tcl/Tk version 8.3, patchlevel 8.3.3.  This is experimental software and probably contains defects. It remains under development. When available, newer versions of tk75 will be made available for download at this web site.

- Downloading and Installing tk75 for Windows

- Download and run the free tcl/tk software from: http://prdownloads.sourceforge.net/tcl/tcl832.exe .

It will ask you to choose a folder in which to install the tcl/tk software. You must install it in a new folder named Tcl on the C partition:

C:\Tcl

- Create another installation folder, named C:\tk75. It will be used for the tk75 program and data files.

- Download the zipped version of tk75, tk75-04.zip into the C:\tk75 folder you created earlier.

- Unzip the file into the C:\tk75 folder after downloading it. (Note to Windows XP users: Be careful to extract the zip file into the correct folder. Windows XP defaults to the wrong folder and you must explicitly choose the correct folder.)

- Remove the tk75.ini configuration file if you have one left over from a previous installation.

- You must reboot Windows before using tk75 for the first time.

- To start tk75, run the C:\tk75\tk75.bat batch file.

- Notes

- This installation procedure changed and is now the same for Windows XP, Windows 98, and Windows ME. The older instructions required editing the autoexec.bat file, a file not used in Windows XP.

- If you create a shortcut to C:\tk75\tk75.bat, the shortcut works best if it is set up to run minimized and close on exit.

- Before using tk75, be sure the IC-R75's "initial set mode" options are set as follows:

  - CIV ADDR = 5AH

  - CIV BAUD = AUTO

  - CIV TRAN = OFF

  - CIV 731 = OFF

See pp. 30 - 33 in the IC-R75 instruction manual to learn how to set them.

- Power your IC-R75 receiver on before starting tk75 or else the program will freeze.

- Overview

- tk75 is open source software designed to control and program the ICOM IC-R75 receiver. An introductory article about this software was published in September 2002 Monitoring Times magazine.

- I implemented tk75 in the Tcl/Tk scripting language. The free Tcl/Tk interpreter software must be installed on your computer prior to using tk75. This enables tk75 to run on a variety of operating systems, including Linux , openBSD, Microsoft Windows, MacOS X, and Sun Solaris. I have not tested it on other platforms, but it will probably work on them after little or no modification.

- tk75 is distributed free of charge, but it is neither shareware nor in the public domain. tk75 is a copyrighted work released under the terms of the GNU General Public License as published by the Free Software Foundation. tk75 is distributed in the hope that it will be useful, but WITHOUT ANY WARRANTY; without even the implied warranty of MERCHANTABILITY or FITNESS FOR A PARTICULAR PURPOSE. See the GNU General Public License for more details.

- Most IC-R75 controls are available on tk75's Main Controls window and Secondary Controls window . Less frequently used parameters may be accessed using the pulldown menus, which may be "torn off" and used as a separate window. tk75 provides control of virtually every Set Mode adjustment. A filter menu and filter configuration applet manages installation of optional filters and clearly associates IF filters with wide, medium, and narrow selectivity settings for each mode.

- tk75 lets you scan memory channels and perform limit searches.

- tk75 can be used to load the IC-R75 memories or read memory data from the radio and store it in a file.

- Data files are in csv (comma-separated values) format, so users can create or edit them using spreadsheet or text editor programs.

- tk75 software provides a facility to log and timestamp active frequencies and relative signal strengths to a log file in csv format. For example:

2002/05/10,19:22:27,5.000000,AM,0.00,"WWV"

2002/05/10,19:23:37,49.875000,FM,4.53,"BABYM"

- Memory Channel Files

- tk75 can read files containing memory channel data. Using the mouse, you can click on a particular entry and tk75 will tune the radio and set the mode, skip and select flags accordingly.

- You can also download the memory data from a file to the IC-R75, thus programming its memories.

- A memory channel data file must be formated in csv (comma-separated values) format. The first line of the file should be this heading:

- Ch,MHz,Mode,Filter,Label,Select,Preamp,Att,Ant

- All the other lines should contain these fields:

Channel number: 1 - 99

Frequency in MHz

Mode: am, sam, usb, lsb, cw, cwr, fm, rtty, rttyr

Selectivity: wide, medium, narrow

select (optional)

Preamplifier (optional): preamp1, preamp2

Attenuation (optional): atten

Antenna (optional): ant1 ant2

Label (optional): 8-char name which should

not contain a comma or quotation marks.

This is a sample memory data file:

Ch,MHz,Mode,Selectivity,Label,Select,Preamp,Att,Ant

1,0.6,AM,WIDE,"WMT Ceda",,,,

2,0.62,AM,WIDE,"WTMJ Mil",,,,

3,0.72,AM,WIDE,"WGN Chic",select,preamp1,,ant2

4,0.78,AM,WIDE,"WBBM Chi",select,,atten,ant2

5,0.85,AM,WIDE,"WAIT Cry",,preamp2,,ant1

6,0.89,AM,WIDE,"WLS Chic",select,,,

7,1.14,AM,WIDE,"WRVA Ric",,,,

8,1.16,AM,WIDE,"WYLL Chi",,,,

9,1.19,AM,WIDE,"WOWO Ft.",,,,

10,1.22,AM,WIDE,"WLPO Las",,,,

19,1.12,AM,WIDE,"KMOX St.",,,,

You can use a separate text editor or spreadsheet program to create, edit, and print memory channel files. You cannot create or edit memory channel data files within tk75.

- Downloading and Installing tk5 
on Windows Based Systems

Version 0.6

This is experimental software and contains defects. It remains under development. When available, newer versions of tk5 will be made available for download at this web site.

- Downloading and Installing tk5 for Windows

- You need to download and install version 8.4.1 of the free Tcl/Tk software. If you have an older version already on your system, you must uninstall the older version first.

Download and run the free Tcl/Tk software from: http://downloads.activestate.com/ActiveTcl/Windows/8.4.1/ActiveTcl8.4.1.0-2-win32-ix86.exe . (The file size is about 11.2 MB.)

It will ask you to choose a directory in which to install the Tcl/Tk software. You must install it in a new folder named Tcl on the C drive, i.e.,

C:\Tcl

- Create another installation folder, named C:\tk5. It will be used for the tk5 program and data files.

- Download the zipped version of tk5, tk5-06.zip into the C:\tk5 folder you created earlier.

- Unzip the file into the C:\tk5 folder after downloading it. (Note to Windows XP users: Be careful to extract the zip file into the correct folder. Windows XP defaults to the wrong folder and you must explicitly choose the correct folder.)

- Remove the tk5.ini configuration file if you have one left over from a previous installation.

- You must reboot Windows before using tk5 for the first time.

- To start tk5, run the C:\tk5\tk5.bat batch file.

- Notes

- This installation procedure is the same for Windows XP, Windows 98, and Windows ME.

- If you create a shortcut to C:\tk5\tk5.bat, the shortcut works best if it is set up to run minimized and close on exit.

- Be sure to read a memory image from your radio and save it in a template (.tr5) file as a backup before making any changes.